/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Dual HC-SR04 Boundary Measurement + I2C LCD Display

*

* SYSCLK = 168 MHz | APB1 timer clock = 84 MHz

*

* SENSOR 1 (Horizontal: left/right)  TRIG=PC6    ECHO=PC7

* SENSOR 2 (Vertical:   bottom/top)  TRIG=PB12   ECHO=PB5

*

* BUTTONS:

* External Button → PA3  (Wired to 3.3V, Rising-edge EXTI, Internal Pulldown)

* (Handles mode progression and lodging values)

*

* LEDs:

* LD1 Green PB0  → ON while in measurement mode, OFF in idle

* LD2 Blue  PB7  → flashes twice when a value is lodged

* LD3 Red   PB14 → reserved

*

* I2C LCD 16x2 (PCF8574 backpack, addr 0x27):

* I2C1  SCL=PB8   SDA=PB9

* Row 0: "L:xx.x   R:xx.x  "

* Row 1: "B:xx.x   T:xx.x  "

* Unset values display as "--.-"

*

* TIMERS:

* TIM2  Prescaler=83, Period=0xFFFFFFFF  1µs free-run  (HC-SR04 timing)

* TIM3  Prescaler=8399, Period=999       10ms IRQ tick (LCD refresh)

*

* STATE MACHINE (Single Button Progression):

* MODE 0  IDLE    LD1 OFF.  Press PA3 Button → MODE 1

* MODE 1  LEFT    Sensor 1. Press PA3 Button → store dist_left  → MODE 2

* MODE 2  RIGHT   Sensor 1. Press PA3 Button → store dist_right → MODE 3

* MODE 3  BOTTOM  Sensor 2. Press PA3 Button → store dist_bot   → MODE 4

* MODE 4  TOP     Sensor 2. Press PA3 Button → store dist_top   → MODE 0

******************************************************************************

*/

/* USER CODE END Header */

/* Includes ------------------------------------------------------------------*/

#include "main.h"

/* Private includes ----------------------------------------------------------*/

/* USER CODE BEGIN Includes */

#include <string.h>

#include <stdio.h>

/* USER CODE END Includes */

/* Private typedef -----------------------------------------------------------*/

/* USER CODE BEGIN PTD */

/* USER CODE END PTD */

/* Private define ------------------------------------------------------------*/

/* USER CODE BEGIN PD */

#define DEBOUNCE_DELAY_MS 50   /* 250ms debounce window for button */

/* External Button Definitions */

#define BTN_EXT_Pin       GPIO_PIN_3

#define BTN_EXT_GPIO_Port GPIOA

/* USER CODE END PD */

/* Private macro -------------------------------------------------------------*/

/* USER CODE BEGIN PM */

/* USER CODE END PM */

/* Private variables ---------------------------------------------------------*/

I2C_HandleTypeDef hi2c1;

TIM_HandleTypeDef htim2;

UART_HandleTypeDef huart3;

PCD_HandleTypeDef hpcd_USB_OTG_FS;

/* USER CODE BEGIN PV */

TIM_HandleTypeDef htim3; /* Manually added for LCD refresh 10ms IRQ */

static float dist_left  = -1.0f;

static float dist_right = -1.0f;

static float dist_bot   = -1.0f;

static float dist_top   = -1.0f;

static volatile uint8_t systemMode     = 0;

static volatile uint8_t flag_userBtn   = 0;   /* set by EXTI3 ISR           */

static volatile uint8_t flag_lcdRefresh= 0;   /* set by TIM3 every 100 ms   */

/* USER CODE END PV */

/* Private function prototypes -----------------------------------------------*/

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_USART3_UART_Init(void);

static void MX_USB_OTG_FS_PCD_Init(void);

static void MX_I2C1_Init(void);

static void MX_TIM2_Init(void);

/* USER CODE BEGIN PFP */

static void MX_TIM3_Init(void);

static float HC_SR04_Read(GPIO_TypeDef *tPort, uint16_t tPin,

GPIO_TypeDef *ePort, uint16_t ePin);

static void  LED_Flash(uint16_t pin);

static void  EnterMode(uint8_t mode);

/* LCD */

#define LCD_ADDR      (0x27 << 1)

#define LCD_BACKLIGHT  0x08

static void lcd_send_cmd(char cmd);

static void lcd_send_data(char data);

static void lcd_send_string(char *str);

static void lcd_put_cur(int row, int col);

static void lcd_clear(void);

static void lcd_init(void);

static void lcd_update(void);

/* USER CODE END PFP */

/* Private user code ---------------------------------------------------------*/

/* USER CODE BEGIN 0 */

/* printf → USART3 TX PD8 → ST-Link → CubeIDE terminal at 115200 baud */

int _write(int file, char *ptr, int len)

{

HAL_UART_Transmit(&huart3, (uint8_t *)ptr, len, HAL_MAX_DELAY);

return len;

}

/* ── EXTI: Callback for PA3 (External Button) with Debounce ─────────────── */

void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

if (GPIO_Pin == BTN_EXT_Pin) {

static uint32_t last_interrupt_time = 0;

uint32_t interrupt_time = HAL_GetTick();

/* Check if enough time has passed since the last valid interrupt */

if (interrupt_time - last_interrupt_time > DEBOUNCE_DELAY_MS) {

flag_userBtn = 1;

last_interrupt_time = interrupt_time;

}

}

}

/* ── TIM3 period elapsed → LCD refresh flag every 100 ms ────────────────── */

void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim)

{

static uint8_t ticks = 0;

if (htim->Instance == TIM3) {

if (++ticks >= 10) {   /* 10 × 10 ms = 100 ms */

ticks = 0;

flag_lcdRefresh = 1;

}

}

}

/* ── HC-SR04 distance measurement (cm) ────────────────────────────────────── */

static float HC_SR04_Read(GPIO_TypeDef *tPort, uint16_t tPin,

GPIO_TypeDef *ePort, uint16_t ePin)

{

uint32_t t;

/* TRIG LOW 5 µs */

HAL_GPIO_WritePin(tPort, tPin, GPIO_PIN_RESET);

t = __HAL_TIM_GET_COUNTER(&htim2);

while ((__HAL_TIM_GET_COUNTER(&htim2) - t) < 5U);

/* TRIG HIGH 10 µs */

HAL_GPIO_WritePin(tPort, tPin, GPIO_PIN_SET);

t = __HAL_TIM_GET_COUNTER(&htim2);

while ((__HAL_TIM_GET_COUNTER(&htim2) - t) < 10U);

HAL_GPIO_WritePin(tPort, tPin, GPIO_PIN_RESET);

/* Wait ECHO HIGH, 30 ms timeout */

uint32_t deadline = HAL_GetTick() + 30UL;

while (HAL_GPIO_ReadPin(ePort, ePin) == GPIO_PIN_RESET)

if (HAL_GetTick() > deadline) return -1.0f;

uint32_t startUs = __HAL_TIM_GET_COUNTER(&htim2);

/* Wait ECHO LOW, 30 ms timeout */

deadline = HAL_GetTick() + 30UL;

while (HAL_GPIO_ReadPin(ePort, ePin) == GPIO_PIN_SET)

if (HAL_GetTick() > deadline) return -1.0f;

return (float)(__HAL_TIM_GET_COUNTER(&htim2) - startUs) / 5800.0f;

}

/* ── Flash LD2 Blue (PB7) twice to confirm value lodged ─────────────────── */

static void LED_Flash(uint16_t pin)

{

for (int i = 0; i < 2; i++) {

HAL_GPIO_WritePin(GPIOB, pin, GPIO_PIN_SET);

HAL_Delay(150);

HAL_GPIO_WritePin(GPIOB, pin, GPIO_PIN_RESET);

HAL_Delay(100);

}

}

/* ── LCD low-level driver (PCF8574 I2C backpack, 4-bit mode) ────────────── */

static void lcd_send_cmd(char cmd)

{

uint8_t buf[4];

buf[0] = (cmd & 0xF0)        | 0x0C | LCD_BACKLIGHT;

buf[1] = (cmd & 0xF0)        | 0x08 | LCD_BACKLIGHT;

buf[2] = ((cmd << 4) & 0xF0) | 0x0C | LCD_BACKLIGHT;

buf[3] = ((cmd << 4) & 0xF0) | 0x08 | LCD_BACKLIGHT;

HAL_I2C_Master_Transmit(&hi2c1, LCD_ADDR, buf, 4, 100);

}

static void lcd_send_data(char data)

{

uint8_t buf[4];

buf[0] = (data & 0xF0)        | 0x0D | LCD_BACKLIGHT;

buf[1] = (data & 0xF0)        | 0x09 | LCD_BACKLIGHT;

buf[2] = ((data << 4) & 0xF0) | 0x0D | LCD_BACKLIGHT;

buf[3] = ((data << 4) & 0xF0) | 0x09 | LCD_BACKLIGHT;

HAL_I2C_Master_Transmit(&hi2c1, LCD_ADDR, buf, 4, 100);

}

static void lcd_put_cur(int row, int col)

{

lcd_send_cmd((row == 0 ? 0x80 : 0xC0) | col);

}

static void lcd_send_string(char *str)

{

while (*str) lcd_send_data(*str++);

}

static void lcd_clear(void)

{

lcd_send_cmd(0x01);

HAL_Delay(2);

}

static void lcd_init(void)

{

HAL_Delay(50);

lcd_send_cmd(0x30); HAL_Delay(5);

lcd_send_cmd(0x30); HAL_Delay(1);

lcd_send_cmd(0x30); HAL_Delay(10);

lcd_send_cmd(0x20);          /* 4-bit mode          */

lcd_send_cmd(0x28);          /* 2 lines, 5x8 font   */

lcd_send_cmd(0x0C);          /* display ON, cursor OFF */

lcd_send_cmd(0x06);          /* entry mode: increment  */

lcd_send_cmd(0x01);          /* clear display          */

HAL_Delay(2);

}

/* ── LCD update ────────────────────────────────────────────────────────────── */

static void lcd_update(void)

{

char row0[24], row1[24];

char lS[7], rS[7], bS[7], tS[7];

#define GET_FRAC(val) (int)(((val) - (int)(val)) * 10.0f)

if (dist_left < 0)  snprintf(lS, 7, "--.-");

else snprintf(lS, 7, "%d.%d", (int)dist_left,  GET_FRAC(dist_left));

if (dist_right < 0) snprintf(rS, 7, "--.-");

else snprintf(rS, 7, "%d.%d", (int)dist_right, GET_FRAC(dist_right));

if (dist_bot < 0)   snprintf(bS, 7, "--.-");

else snprintf(bS, 7, "%d.%d", (int)dist_bot,   GET_FRAC(dist_bot));

if (dist_top < 0)   snprintf(tS, 7, "--.-");

else snprintf(tS, 7, "%d.%d", (int)dist_top,   GET_FRAC(dist_top));

snprintf(row0, sizeof(row0), "L:%-4s   R:%-4s", lS, rS);

snprintf(row1, sizeof(row1), "B:%-4s   T:%-4s", bS, tS);

for (int i = strlen(row0); i < 16; i++) { row0[i] = ' '; } row0[16] = '\0';

for (int i = strlen(row1); i < 16; i++) { row1[i] = ' '; } row1[16] = '\0';

lcd_put_cur(0, 0); lcd_send_string(row0);

lcd_put_cur(1, 0); lcd_send_string(row1);

}

/* ── EnterMode: set LD1, print to console, refresh LCD ──────────────────── */

static void EnterMode(uint8_t mode)

{

systemMode = mode;

if (mode == 0) {

HAL_GPIO_WritePin(GPIOB, LD1_Pin, GPIO_PIN_RESET);

printf("\r\n[MODE 0] IDLE — LD1 OFF\r\n");

printf("Press Button (PA3) to begin session.\r\n\r\n");

} else {

HAL_GPIO_WritePin(GPIOB, LD1_Pin, GPIO_PIN_SET);

switch (mode) {

case 1: printf("\r\n[MODE 1] Move SENSOR 1 to LEFT-MOST.   Press Button to lodge.\r\n"); break;

case 2: printf("\r\n[MODE 2] Move SENSOR 1 to RIGHT-MOST.  Press Button to lodge.\r\n"); break;

case 3: printf("\r\n[MODE 3] Move SENSOR 2 to BOTTOM-MOST. Press Button to lodge.\r\n"); break;

case 4: printf("\r\n[MODE 4] Move SENSOR 2 to TOP-MOST.    Press Button to lodge.\r\n"); break;

}

}

lcd_update();

}

static void MX_TIM3_Init(void)

{

TIM_ClockConfigTypeDef  sClk = {0};

TIM_MasterConfigTypeDef sMst = {0};

__HAL_RCC_TIM3_CLK_ENABLE();

htim3.Instance               = TIM3;

htim3.Init.Prescaler         = 8400 - 1;

htim3.Init.CounterMode       = TIM_COUNTERMODE_UP;

htim3.Init.Period            = 1000 - 1;

htim3.Init.ClockDivision     = TIM_CLOCKDIVISION_DIV1;

htim3.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

if (HAL_TIM_Base_Init(&htim3) != HAL_OK) Error_Handler();

sClk.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim3, &sClk) != HAL_OK) Error_Handler();

sMst.MasterOutputTrigger = TIM_TRGO_RESET;

sMst.MasterSlaveMode     = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim3, &sMst) != HAL_OK) Error_Handler();

HAL_NVIC_SetPriority(TIM3_IRQn, 2, 0);

HAL_NVIC_EnableIRQ(TIM3_IRQn);

}

/* USER CODE END 0 */

/**

* @brief  The application entry point.

* @retval int

*/

int main(void)

{

/* MCU Configuration--------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

/* Initialize all configured peripherals */

MX_GPIO_Init();

MX_USART3_UART_Init();

MX_USB_OTG_FS_PCD_Init();

MX_I2C1_Init();

MX_TIM2_Init();

/* USER CODE BEGIN 2 */

MX_TIM3_Init();

/* Enable EXTI Line 3 for PA3 */

HAL_NVIC_SetPriority(EXTI3_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI3_IRQn);

HAL_TIM_Base_Start(&htim2);

HAL_TIM_Base_Start_IT(&htim3);

HAL_GPIO_WritePin(GPIOB, LD1_Pin | LD2_Pin | LD3_Pin, GPIO_PIN_RESET);

lcd_init();

lcd_clear();

printf("==========================================\r\n");

printf("  Dual HC-SR04 Boundary Measurement      \r\n");

printf("  Sensor 1: TRIG=PC6   ECHO=PC7          \r\n");

printf("  Sensor 2: TRIG=PB12  ECHO=PB5          \r\n");

printf("  Control:  EXT_BTN (PA3) -> 3.3V        \r\n");

printf("  LCD I2C1: SCL=PB8   SDA=PB9            \r\n");

printf("==========================================\r\n");

EnterMode(0);

/* USER CODE END 2 */

/* Infinite loop */

/* USER CODE BEGIN WHILE */

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

if (flag_lcdRefresh) {

flag_lcdRefresh = 0;

lcd_update();

}

if (flag_userBtn) {

flag_userBtn = 0;

float reading = -1.0f;

switch (systemMode) {

case 0:

dist_left = dist_right = dist_bot = dist_top = -1.0f;

EnterMode(1);

break;

case 1:

reading = HC_SR04_Read(GPIOC, GPIO_PIN_6, GPIOC, GPIO_PIN_7);

if (reading < 0.0f) {

printf("[MODE 1] Sensor 1 timeout — check wiring, retry.\r\n");

} else {

dist_left = reading;

printf("[MODE 1] LEFT  lodged: %d.%d cm\r\n", (int)dist_left, (int)((dist_left - (int)dist_left) * 10));

LED_Flash(LD2_Pin);

EnterMode(2);

}

break;

case 2:

reading = HC_SR04_Read(GPIOC, GPIO_PIN_6, GPIOC, GPIO_PIN_7);

if (reading < 0.0f) {

printf("[MODE 2] Sensor 1 timeout — retry.\r\n");

} else {

dist_right = reading;

printf("[MODE 2] RIGHT lodged: %d.%d cm\r\n", (int)dist_right, (int)((dist_right - (int)dist_right) * 10));

LED_Flash(LD2_Pin);

EnterMode(3);

}

break;

case 3:

reading = HC_SR04_Read(GPIOB, GPIO_PIN_12, GPIOB, GPIO_PIN_5);

if (reading < 0.0f) {

printf("[MODE 3] Sensor 2 timeout — retry.\r\n");

} else {

dist_bot = reading;

printf("[MODE 3] BOTTOM lodged: %d.%d cm\r\n", (int)dist_bot, (int)((dist_bot - (int)dist_bot) * 10));

LED_Flash(LD2_Pin);

EnterMode(4);

}

break;

case 4:

reading = HC_SR04_Read(GPIOB, GPIO_PIN_12, GPIOB, GPIO_PIN_5);

if (reading < 0.0f) {

printf("[MODE 4] Sensor 2 timeout — retry.\r\n");

} else {

dist_top = reading;

printf("[MODE 4] TOP lodged: %d.%d cm\r\n", (int)dist_top, (int)((dist_top - (int)dist_top) * 10));

LED_Flash(LD2_Pin);

float width = dist_right - dist_left;

float height = dist_top - dist_bot;

printf("\r\n==========================================\r\n");

printf("  CAPTURE COMPLETE                        \r\n");

printf("  LEFT   = %d.%d cm\r\n", (int)dist_left, (int)((dist_left - (int)dist_left) * 10));

printf("  RIGHT  = %d.%d cm\r\n", (int)dist_right, (int)((dist_right - (int)dist_right) * 10));

printf("  BOTTOM = %d.%d cm\r\n", (int)dist_bot, (int)((dist_bot - (int)dist_bot) * 10));

printf("  TOP    = %d.%d cm\r\n", (int)dist_top, (int)((dist_top - (int)dist_top) * 10));

printf("  Width  = %d.%d cm\r\n", (int)width, (int)((width - (int)width) * 10));

printf("  Height = %d.%d cm\r\n", (int)height, (int)((height - (int)height) * 10));

printf("==========================================\r\n");

EnterMode(0);

}

break;

}

}

/* USER CODE END 3 */

}

}

/**

* @brief System Clock Configuration

*/

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE;

RCC_OscInitStruct.HSEState = RCC_HSE_BYPASS;

RCC_OscInitStruct.PLL.PLLState = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource = RCC_PLLSOURCE_HSE;

RCC_OscInitStruct.PLL.PLLM = 4;

RCC_OscInitStruct.PLL.PLLN = 168;

RCC_OscInitStruct.PLL.PLLP = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLQ = 7;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK) Error_Handler();

RCC_ClkInitStruct.ClockType = RCC_CLOCKTYPE_HCLK|RCC_CLOCKTYPE_SYSCLK

|RCC_CLOCKTYPE_PCLK1|RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5) != HAL_OK) Error_Handler();

}

static void MX_I2C1_Init(void)

{

hi2c1.Instance = I2C1;

hi2c1.Init.ClockSpeed = 100000;

hi2c1.Init.DutyCycle = I2C_DUTYCYCLE_2;

hi2c1.Init.OwnAddress1 = 0;

hi2c1.Init.AddressingMode = I2C_ADDRESSINGMODE_7BIT;

hi2c1.Init.DualAddressMode = I2C_DUALADDRESS_DISABLE;

hi2c1.Init.OwnAddress2 = 0;

hi2c1.Init.GeneralCallMode = I2C_GENERALCALL_DISABLE;

hi2c1.Init.NoStretchMode = I2C_NOSTRETCH_DISABLE;

if (HAL_I2C_Init(&hi2c1) != HAL_OK) Error_Handler();

}

static void MX_TIM2_Init(void)

{

TIM_ClockConfigTypeDef sClockSourceConfig = {0};

TIM_MasterConfigTypeDef sMasterConfig = {0};

htim2.Instance = TIM2;

htim2.Init.Prescaler = 0;

htim2.Init.CounterMode = TIM_COUNTERMODE_UP;

htim2.Init.Period = 4294967295;

htim2.Init.ClockDivision = TIM_CLOCKDIVISION_DIV1;

htim2.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

if (HAL_TIM_Base_Init(&htim2) != HAL_OK) Error_Handler();

sClockSourceConfig.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim2, &sClockSourceConfig) != HAL_OK) Error_Handler();

sMasterConfig.MasterOutputTrigger = TIM_TRGO_RESET;

sMasterConfig.MasterSlaveMode = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim2, &sMasterConfig) != HAL_OK) Error_Handler();

}

static void MX_USART3_UART_Init(void)

{

huart3.Instance = USART3;

huart3.Init.BaudRate = 115200;

huart3.Init.WordLength = UART_WORDLENGTH_8B;

huart3.Init.StopBits = UART_STOPBITS_1;

huart3.Init.Parity = UART_PARITY_NONE;

huart3.Init.Mode = UART_MODE_TX_RX;

huart3.Init.HwFlowCtl = UART_HWCONTROL_NONE;

huart3.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart3) != HAL_OK) Error_Handler();

}

static void MX_USB_OTG_FS_PCD_Init(void)

{

hpcd_USB_OTG_FS.Instance = USB_OTG_FS;

hpcd_USB_OTG_FS.Init.dev_endpoints = 4;

hpcd_USB_OTG_FS.Init.speed = PCD_SPEED_FULL;

hpcd_USB_OTG_FS.Init.dma_enable = DISABLE;

hpcd_USB_OTG_FS.Init.phy_itface = PCD_PHY_EMBEDDED;

hpcd_USB_OTG_FS.Init.Sof_enable = ENABLE;

hpcd_USB_OTG_FS.Init.low_power_enable = DISABLE;

hpcd_USB_OTG_FS.Init.lpm_enable = DISABLE;

hpcd_USB_OTG_FS.Init.vbus_sensing_enable = ENABLE;

hpcd_USB_OTG_FS.Init.use_dedicated_ep1 = DISABLE;

if (HAL_PCD_Init(&hpcd_USB_OTG_FS) != HAL_OK) Error_Handler();

}

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

/* GPIO Ports Clock Enable */

__HAL_RCC_SYSCFG_CLK_ENABLE();

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOG_CLK_ENABLE();

HAL_GPIO_WritePin(GPIOB, LD1_Pin|LD3_Pin|GPIO_PIN_12|LD2_Pin, GPIO_PIN_RESET);

HAL_GPIO_WritePin(USB_PowerSwitchOn_GPIO_Port, USB_PowerSwitchOn_Pin, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_6, GPIO_PIN_RESET);

/* Configure external button on PA3 */

GPIO_InitStruct.Pin = BTN_EXT_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_IT_FALLING;

GPIO_InitStruct.Pull = GPIO_PULLUP;     /* Internal pull-down enabled */

HAL_GPIO_Init(BTN_EXT_GPIO_Port, &GPIO_InitStruct);

/* Configure RMII/Ethernet pins */

GPIO_InitStruct.Pin = RMII_MDC_Pin|RMII_RXD1_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_AF_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Alternate = GPIO_AF11_ETH;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

GPIO_InitStruct.Pin = RMII_REF_CLK_Pin|RMII_MDIO_Pin|RMII_CRS_DV_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_AF_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Alternate = GPIO_AF11_ETH;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

GPIO_InitStruct.Pin = GPIO_PIN_4|GPIO_PIN_7;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_NOPULL;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

GPIO_InitStruct.Pin = LD1_Pin|LD3_Pin|GPIO_PIN_12|LD2_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

GPIO_InitStruct.Pin = RMII_TXD1_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_AF_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Alternate = GPIO_AF11_ETH;

HAL_GPIO_Init(RMII_TXD1_GPIO_Port, &GPIO_InitStruct);

GPIO_InitStruct.Pin = USB_PowerSwitchOn_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(USB_PowerSwitchOn_GPIO_Port, &GPIO_InitStruct);

GPIO_InitStruct.Pin = USB_OverCurrent_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_NOPULL;

HAL_GPIO_Init(USB_OverCurrent_GPIO_Port, &GPIO_InitStruct);

GPIO_InitStruct.Pin = GPIO_PIN_6;

GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

GPIO_InitStruct.Pin = RMII_TX_EN_Pin|RMII_TXD0_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_AF_PP;

GPIO_InitStruct.Pull = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Alternate = GPIO_AF11_ETH;

HAL_GPIO_Init(GPIOG, &GPIO_InitStruct);

GPIO_InitStruct.Pin = GPIO_PIN_5;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_NOPULL;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

}

/* USER CODE BEGIN 4 */

/* Update to use EXTI3 for PA3 */

void EXTI3_IRQHandler(void) { HAL_GPIO_EXTI_IRQHandler(BTN_EXT_Pin); }

void TIM3_IRQHandler(void)  { HAL_TIM_IRQHandler(&htim3);            }

/* USER CODE END 4 */

void Error_Handler(void)

{

__disable_irq();

while (1) { }

}