/* USER CODE BEGIN Header */

/**

******************************************************************************

* @file           : main.c

* @brief          : Dual HC-SR04 Boundary Measurement + I2C LCD Display

*

* SYSCLK = 168 MHz | APB1 timer clock = 84 MHz

*

* SENSOR 1 (Horizontal: left/right)  TRIG=PC6   ECHO=PC7

* SENSOR 2 (Vertical:   bottom/top)  TRIG=PB12  ECHO=PB5

*

* BUTTON:

*   PA3 → wired to 3.3V, Falling-edge EXTI, internal Pull-Up

*   Single button controls all mode progression AND lodging.

*

* KEY DESIGN — TWO-STAGE READ:

*   Stage A (ISR):  Button press sets flag_doRead immediately and returns.

*                   ISR is NEVER blocked. Button is always responsive.

*   Stage B (loop): Main loop sees flag_doRead, calls HC_SR04_Read(),

*                   lodges value, advances mode.

*   This means the ~30 ms ultrasonic read happens in the main loop

*   where blocking is acceptable, and the button ISR stays fast.

*

* LEDs:

*   LD1 Green PB0  → ON in measurement mode, OFF in idle

*   LD2 Blue  PB7  → flashes twice when a value is lodged

*

* LCD 16x2 I2C (PCF8574, 0x27):

*   SCL=PB8  SDA=PB9

*   Row 0: "L:xx.x   R:xx.x"

*   Row 1: "B:xx.x   T:xx.x"

*

* TIMERS:

*   TIM2  Prescaler=83, free-run  → 1 µs tick for HC-SR04

*   TIM3  Prescaler=8399, 10ms   → LCD refresh flag

******************************************************************************

*/

/* USER CODE END Header */

#include "main.h"

#include <string.h>

#include <stdio.h>

/* ── Handles ─────────────────────────────────────────────────────────────── */

I2C_HandleTypeDef  hi2c1;

TIM_HandleTypeDef  htim2;

UART_HandleTypeDef huart3;

PCD_HandleTypeDef  hpcd_USB_OTG_FS;

/* USER CODE BEGIN PV */

TIM_HandleTypeDef htim3;

/* Boundary distances in cm. -1 = not yet captured */

static float dist_left  = -1.0f;

static float dist_right = -1.0f;

static float dist_bot   = -1.0f;

static float dist_top   = -1.0f;

static volatile uint8_t systemMode     = 0;

/* ── TWO FLAGS — keep them separate ────────────────────────────────────────

flag_doRead : set by ISR  → tells main loop to perform a sensor read

cleared by main loop after read completes

flag_lcdRefresh : set by TIM3 → tells main loop to refresh LCD          */

static volatile uint8_t flag_doRead    = 0;

static volatile uint8_t flag_lcdRefresh= 0;

/* USER CODE END PV */

/* ── Defines ─────────────────────────────────────────────────────────────── */

#define DEBOUNCE_MS       200

#define BTN_EXT_Pin       GPIO_PIN_3

#define BTN_EXT_GPIO_Port GPIOA

/* ── Prototypes ──────────────────────────────────────────────────────────── */

void SystemClock_Config(void);

static void MX_GPIO_Init(void);

static void MX_USART3_UART_Init(void);

static void MX_USB_OTG_FS_PCD_Init(void);

static void MX_I2C1_Init(void);

static void MX_TIM2_Init(void);

/* USER CODE BEGIN PFP */

static void  MX_TIM3_Init(void);

static float HC_SR04_Read(GPIO_TypeDef *tPort, uint16_t tPin,

GPIO_TypeDef *ePort, uint16_t ePin);

static void  LED_Flash(uint16_t pin);

static void  EnterMode(uint8_t mode);

static void  DoSensorReadAndLodge(void);

#define LCD_ADDR      (0x27 << 1)

#define LCD_BACKLIGHT  0x08

static void lcd_send_cmd(char cmd);

static void lcd_send_data(char data);

static void lcd_send_string(char *str);

static void lcd_put_cur(int row, int col);

static void lcd_clear(void);

static void lcd_init(void);

static void lcd_update(void);

/* USER CODE END PFP */

/* ═══════════════════════════════════════════════════════════════════════════

USER CODE BEGIN 0

═══════════════════════════════════════════════════════════════════════════ */

int _write(int file, char *ptr, int len)

{

HAL_UART_Transmit(&huart3, (uint8_t *)ptr, len, HAL_MAX_DELAY);

return len;

}

/* ── EXTI ISR Callback ───────────────────────────────────────────────────────

THIS IS ALL THE ISR DOES:

1. Debounce check

2. Set flag_doRead = 1

3. Return immediately

The sensor read and all HAL_Delay() calls happen in the main loop only.

─────────────────────────────────────────────────────────────────────────── */

void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

if (GPIO_Pin == BTN_EXT_Pin)

{

static uint32_t lastPressTime = 0;

uint32_t now = HAL_GetTick();

if ((now - lastPressTime) > DEBOUNCE_MS)

{

lastPressTime = now;

flag_doRead = 1;   /* just set the flag — do nothing else here */

}

}

}

/* ── TIM3: 10 ms tick → LCD refresh every 100 ms ────────────────────────── */

void HAL_TIM_PeriodElapsedCallback(TIM_HandleTypeDef *htim)

{

static uint8_t ticks = 0;

if (htim->Instance == TIM3)

{

if (++ticks >= 10) {

ticks = 0;

flag_lcdRefresh = 1;

}

}

}

/* ── HC-SR04 ─────────────────────────────────────────────────────────────────

Called ONLY from the main loop — never from an ISR or timer callback.

Blocks for up to ~60 ms (two 30 ms timeouts) in worst case.

This is fine in the main loop where nothing time-critical is missed.

─────────────────────────────────────────────────────────────────────────── */

static float HC_SR04_Read(GPIO_TypeDef *tPort, uint16_t tPin,

GPIO_TypeDef *ePort, uint16_t ePin)

{

uint32_t t;

/* TRIG LOW for 5 µs */

HAL_GPIO_WritePin(tPort, tPin, GPIO_PIN_RESET);

t = __HAL_TIM_GET_COUNTER(&htim2);

while ((__HAL_TIM_GET_COUNTER(&htim2) - t) < 5U);

/* TRIG HIGH for 10 µs */

HAL_GPIO_WritePin(tPort, tPin, GPIO_PIN_SET);

t = __HAL_TIM_GET_COUNTER(&htim2);

while ((__HAL_TIM_GET_COUNTER(&htim2) - t) < 10U);

HAL_GPIO_WritePin(tPort, tPin, GPIO_PIN_RESET);

/* Wait for ECHO HIGH — 30 ms timeout */

uint32_t deadline = HAL_GetTick() + 30UL;

while (HAL_GPIO_ReadPin(ePort, ePin) == GPIO_PIN_RESET)

if (HAL_GetTick() > deadline) return -1.0f;

uint32_t startUs = __HAL_TIM_GET_COUNTER(&htim2);

/* Wait for ECHO LOW — 30 ms timeout */

deadline = HAL_GetTick() + 30UL;

while (HAL_GPIO_ReadPin(ePort, ePin) == GPIO_PIN_SET)

if (HAL_GetTick() > deadline) return -1.0f;

/* distance (cm) = pulse_µs / 58 */

return (float)(__HAL_TIM_GET_COUNTER(&htim2) - startUs) / 58.0f;

}

/* ── LED flash — called from main loop only ──────────────────────────────── */

static void LED_Flash(uint16_t pin)

{

for (int i = 0; i < 2; i++) {

HAL_GPIO_WritePin(GPIOB, pin, GPIO_PIN_SET);

HAL_Delay(150);

HAL_GPIO_WritePin(GPIOB, pin, GPIO_PIN_RESET);

HAL_Delay(100);

}

}

/* ── LCD ─────────────────────────────────────────────────────────────────── */

static void lcd_send_cmd(char cmd)

{

uint8_t buf[4];

buf[0] = (cmd  & 0xF0)        | 0x0C | LCD_BACKLIGHT;

buf[1] = (cmd  & 0xF0)        | 0x08 | LCD_BACKLIGHT;

buf[2] = ((cmd << 4) & 0xF0)  | 0x0C | LCD_BACKLIGHT;

buf[3] = ((cmd << 4) & 0xF0)  | 0x08 | LCD_BACKLIGHT;

HAL_I2C_Master_Transmit(&hi2c1, LCD_ADDR, buf, 4, 100);

}

static void lcd_send_data(char data)

{

uint8_t buf[4];

buf[0] = (data  & 0xF0)        | 0x0D | LCD_BACKLIGHT;

buf[1] = (data  & 0xF0)        | 0x09 | LCD_BACKLIGHT;

buf[2] = ((data << 4) & 0xF0)  | 0x0D | LCD_BACKLIGHT;

buf[3] = ((data << 4) & 0xF0)  | 0x09 | LCD_BACKLIGHT;

HAL_I2C_Master_Transmit(&hi2c1, LCD_ADDR, buf, 4, 100);

}

static void lcd_put_cur(int row, int col)

{

lcd_send_cmd((row == 0 ? 0x80 : 0xC0) | col);

}

static void lcd_send_string(char *str)

{

while (*str) lcd_send_data(*str++);

}

static void lcd_clear(void)

{

lcd_send_cmd(0x01);

HAL_Delay(2);

}

static void lcd_init(void)

{

HAL_Delay(50);

lcd_send_cmd(0x30); HAL_Delay(5);

lcd_send_cmd(0x30); HAL_Delay(1);

lcd_send_cmd(0x30); HAL_Delay(10);

lcd_send_cmd(0x20);

lcd_send_cmd(0x28);

lcd_send_cmd(0x0C);

lcd_send_cmd(0x06);

lcd_send_cmd(0x01);

HAL_Delay(2);

}

static void lcd_update(void)

{

char row0[17], row1[17];

char lS[7], rS[7], bS[7], tS[7];

#define FMT(buf, val) \

if ((val) < 0) snprintf(buf, 7, "--.-"); \

else snprintf(buf, 7, "%d.%d", (int)(val), \

(int)(((val) - (int)(val)) * 10.0f))

FMT(lS, dist_left);

FMT(rS, dist_right);

FMT(bS, dist_bot);

FMT(tS, dist_top);

snprintf(row0, 17, "L:%-4s  R:%-4s", lS, rS);

snprintf(row1, 17, "B:%-4s  T:%-4s", bS, tS);

for (int i = strlen(row0); i < 16; i++) row0[i] = ' '; row0[16] = '\0';

for (int i = strlen(row1); i < 16; i++) row1[i] = ' '; row1[16] = '\0';

lcd_put_cur(0, 0); lcd_send_string(row0);

lcd_put_cur(1, 0); lcd_send_string(row1);

}

static void EnterMode(uint8_t mode)

{

systemMode = mode;

HAL_GPIO_WritePin(GPIOB, LD1_Pin,

(mode == 0) ? GPIO_PIN_RESET : GPIO_PIN_SET);

switch (mode) {

case 0:

printf("\r\n[MODE 0] IDLE — LD1 OFF\r\n");

printf("Press button (PA3) to begin session.\r\n\r\n");

break;

case 1:

printf("\r\n[MODE 1] Sensor 1 → LEFT-MOST.   Press button to read & lodge.\r\n");

break;

case 2:

printf("\r\n[MODE 2] Sensor 1 → RIGHT-MOST.  Press button to read & lodge.\r\n");

break;

case 3:

printf("\r\n[MODE 3] Sensor 2 → BOTTOM-MOST. Press button to read & lodge.\r\n");

break;

case 4:

printf("\r\n[MODE 4] Sensor 2 → TOP-MOST.    Press button to read & lodge.\r\n");

break;

}

lcd_update();

}

/* ── DoSensorReadAndLodge ────────────────────────────────────────────────────

Called from the main loop when flag_doRead == 1.

Performs the full read + lodge + mode advance sequence.

This is where the ~30 ms blocking time is spent — safely in the loop.

─────────────────────────────────────────────────────────────────────────── */

static void DoSensorReadAndLodge(void)

{

float reading = -1.0f;

switch (systemMode)

{

/* ── MODE 0: start new session ── */

case 0:

dist_left = dist_right = dist_bot = dist_top = -1.0f;

EnterMode(1);

break;

/* ── MODE 1: LEFT — Sensor 1 ── */

case 1:

printf("[MODE 1] Reading Sensor 1...\r\n");

reading = HC_SR04_Read(GPIOC, GPIO_PIN_6, GPIOC, GPIO_PIN_7);

if (reading < 0.0f) {

printf("[MODE 1] Timeout — check wiring and retry.\r\n");

} else {

dist_left = reading;

printf("[MODE 1] LEFT lodged: %d.%d cm\r\n",

(int)dist_left,

(int)((dist_left - (int)dist_left) * 10));

LED_Flash(LD2_Pin);

EnterMode(2);

}

break;

/* ── MODE 2: RIGHT — Sensor 1 ── */

case 2:

printf("[MODE 2] Reading Sensor 1...\r\n");

reading = HC_SR04_Read(GPIOC, GPIO_PIN_6, GPIOC, GPIO_PIN_7);

if (reading < 0.0f) {

printf("[MODE 2] Timeout — retry.\r\n");

} else {

dist_right = reading;

printf("[MODE 2] RIGHT lodged: %d.%d cm\r\n",

(int)dist_right,

(int)((dist_right - (int)dist_right) * 10));

LED_Flash(LD2_Pin);

EnterMode(3);

}

break;

/* ── MODE 3: BOTTOM — Sensor 2 ── */

case 3:

printf("[MODE 3] Reading Sensor 2...\r\n");

reading = HC_SR04_Read(GPIOB, GPIO_PIN_12, GPIOB, GPIO_PIN_5);

if (reading < 0.0f) {

printf("[MODE 3] Timeout — retry.\r\n");

} else {

dist_bot = reading;

printf("[MODE 3] BOTTOM lodged: %d.%d cm\r\n",

(int)dist_bot,

(int)((dist_bot - (int)dist_bot) * 10));

LED_Flash(LD2_Pin);

EnterMode(4);

}

break;

/* ── MODE 4: TOP — Sensor 2 ── */

case 4:

printf("[MODE 4] Reading Sensor 2...\r\n");

reading = HC_SR04_Read(GPIOB, GPIO_PIN_12, GPIOB, GPIO_PIN_5);

if (reading < 0.0f) {

printf("[MODE 4] Timeout — retry.\r\n");

} else {

dist_top = reading;

printf("[MODE 4] TOP lodged: %d.%d cm\r\n",

(int)dist_top,

(int)((dist_top - (int)dist_top) * 10));

LED_Flash(LD2_Pin);

float width  = dist_right - dist_left;

float height = dist_top   - dist_bot;

printf("\r\n==========================================\r\n");

printf("  CAPTURE COMPLETE\r\n");

printf("  LEFT   = %d.%d cm\r\n", (int)dist_left,  (int)((dist_left  - (int)dist_left)  * 10));

printf("  RIGHT  = %d.%d cm\r\n", (int)dist_right, (int)((dist_right - (int)dist_right) * 10));

printf("  BOTTOM = %d.%d cm\r\n", (int)dist_bot,   (int)((dist_bot   - (int)dist_bot)   * 10));

printf("  TOP    = %d.%d cm\r\n", (int)dist_top,   (int)((dist_top   - (int)dist_top)   * 10));

printf("  Width  = %d.%d cm\r\n", (int)width,      (int)((width      - (int)width)      * 10));

printf("  Height = %d.%d cm\r\n", (int)height,     (int)((height     - (int)height)     * 10));

printf("==========================================\r\n");

/* ---> PASTE THIS RIGHT HERE <--- */

printf("DATA:%d.%d,%d.%d,%d.%d,%d.%d\n",

(int)dist_left,  (int)((dist_left  - (int)dist_left)  * 10),

(int)dist_right, (int)((dist_right - (int)dist_right) * 10),

(int)dist_bot,   (int)((dist_bot   - (int)dist_bot)   * 10),

(int)dist_top,   (int)((dist_top   - (int)dist_top)   * 10));

EnterMode(0);

}

break;

}

}

/* USER CODE END 0 */

/* ═══════════════════════════════════════════════════════════════════════════

main()

═══════════════════════════════════════════════════════════════════════════ */

int main(void)

{

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART3_UART_Init();

MX_USB_OTG_FS_PCD_Init();

MX_I2C1_Init();

MX_TIM2_Init();

/* USER CODE BEGIN 2 */

MX_TIM3_Init();

HAL_NVIC_SetPriority(EXTI3_IRQn, 0, 0);

HAL_NVIC_EnableIRQ(EXTI3_IRQn);

HAL_TIM_Base_Start(&htim2);

HAL_TIM_Base_Start_IT(&htim3);

HAL_GPIO_WritePin(GPIOB, LD1_Pin | LD2_Pin | LD3_Pin, GPIO_PIN_RESET);

lcd_init();

lcd_clear();

printf("==========================================\r\n");

printf("  Dual HC-SR04 Boundary Measurement      \r\n");

printf("  Sensor 1: TRIG=PC6   ECHO=PC7          \r\n");

printf("  Sensor 2: TRIG=PB12  ECHO=PB5          \r\n");

printf("  Button:   PA3                           \r\n");

printf("  LCD I2C1: SCL=PB8    SDA=PB9           \r\n");

printf("==========================================\r\n");

EnterMode(0);

/* USER CODE END 2 */

while (1)

{

/* USER CODE BEGIN 3 */

/* ── LCD refresh (non-blocking, flag driven) ────────────────────── */

if (flag_lcdRefresh) {

flag_lcdRefresh = 0;

lcd_update();

}

/* ── Sensor read + lodge (blocking ~30ms, safe here in main loop) ─

flag_doRead is set by the ISR and immediately cleared here.

Any button press during the read is captured by EXTI as soon as

this read finishes and the loop comes back around.               */

if (flag_doRead) {

flag_doRead = 0;      /* clear BEFORE read so a press during   */

/* the read is caught next loop iteration */

DoSensorReadAndLodge();

}

/* USER CODE END 3 */

}

}

/* ═══════════════════════════════════════════════════════════════════════════

Peripheral Init Functions

═══════════════════════════════════════════════════════════════════════════ */

void SystemClock_Config(void)

{

RCC_OscInitTypeDef RCC_OscInitStruct = {0};

RCC_ClkInitTypeDef RCC_ClkInitStruct = {0};

__HAL_RCC_PWR_CLK_ENABLE();

__HAL_PWR_VOLTAGESCALING_CONFIG(PWR_REGULATOR_VOLTAGE_SCALE1);

RCC_OscInitStruct.OscillatorType = RCC_OSCILLATORTYPE_HSE;

RCC_OscInitStruct.HSEState       = RCC_HSE_BYPASS;

RCC_OscInitStruct.PLL.PLLState   = RCC_PLL_ON;

RCC_OscInitStruct.PLL.PLLSource  = RCC_PLLSOURCE_HSE;

RCC_OscInitStruct.PLL.PLLM       = 4;

RCC_OscInitStruct.PLL.PLLN       = 168;

RCC_OscInitStruct.PLL.PLLP       = RCC_PLLP_DIV2;

RCC_OscInitStruct.PLL.PLLQ       = 7;

if (HAL_RCC_OscConfig(&RCC_OscInitStruct) != HAL_OK) Error_Handler();

RCC_ClkInitStruct.ClockType      = RCC_CLOCKTYPE_HCLK | RCC_CLOCKTYPE_SYSCLK

| RCC_CLOCKTYPE_PCLK1 | RCC_CLOCKTYPE_PCLK2;

RCC_ClkInitStruct.SYSCLKSource   = RCC_SYSCLKSOURCE_PLLCLK;

RCC_ClkInitStruct.AHBCLKDivider  = RCC_SYSCLK_DIV1;

RCC_ClkInitStruct.APB1CLKDivider = RCC_HCLK_DIV4;

RCC_ClkInitStruct.APB2CLKDivider = RCC_HCLK_DIV2;

if (HAL_RCC_ClockConfig(&RCC_ClkInitStruct, FLASH_LATENCY_5) != HAL_OK) Error_Handler();

}

static void MX_I2C1_Init(void)

{

hi2c1.Instance             = I2C1;

hi2c1.Init.ClockSpeed      = 100000;

hi2c1.Init.DutyCycle       = I2C_DUTYCYCLE_2;

hi2c1.Init.OwnAddress1     = 0;

hi2c1.Init.AddressingMode  = I2C_ADDRESSINGMODE_7BIT;

hi2c1.Init.DualAddressMode = I2C_DUALADDRESS_DISABLE;

hi2c1.Init.OwnAddress2     = 0;

hi2c1.Init.GeneralCallMode = I2C_GENERALCALL_DISABLE;

hi2c1.Init.NoStretchMode   = I2C_NOSTRETCH_DISABLE;

if (HAL_I2C_Init(&hi2c1) != HAL_OK) Error_Handler();

if (HAL_I2CEx_ConfigAnalogFilter(&hi2c1, I2C_ANALOGFILTER_ENABLE) != HAL_OK) Error_Handler();

if (HAL_I2CEx_ConfigDigitalFilter(&hi2c1, 0) != HAL_OK) Error_Handler();

}

/* TIM2 — 1 µs free-run for HC-SR04

Prescaler=83: 84 MHz / 84 = 1 MHz → 1 tick = 1 µs                       */

static void MX_TIM2_Init(void)

{

TIM_ClockConfigTypeDef  sClk = {0};

TIM_MasterConfigTypeDef sMst = {0};

htim2.Instance               = TIM2;

htim2.Init.Prescaler         = 83;

htim2.Init.CounterMode       = TIM_COUNTERMODE_UP;

htim2.Init.Period            = 4294967295UL;

htim2.Init.ClockDivision     = TIM_CLOCKDIVISION_DIV1;

htim2.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

if (HAL_TIM_Base_Init(&htim2) != HAL_OK) Error_Handler();

sClk.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim2, &sClk) != HAL_OK) Error_Handler();

sMst.MasterOutputTrigger = TIM_TRGO_RESET;

sMst.MasterSlaveMode     = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim2, &sMst) != HAL_OK) Error_Handler();

}

/* TIM3 — 10 ms interrupt for LCD refresh flag

84 MHz / 8400 / 1000 = 10 Hz → 10 ms per tick                            */

static void MX_TIM3_Init(void)

{

TIM_ClockConfigTypeDef  sClk = {0};

TIM_MasterConfigTypeDef sMst = {0};

__HAL_RCC_TIM3_CLK_ENABLE();

htim3.Instance               = TIM3;

htim3.Init.Prescaler         = 8400 - 1;

htim3.Init.CounterMode       = TIM_COUNTERMODE_UP;

htim3.Init.Period            = 1000 - 1;

htim3.Init.ClockDivision     = TIM_CLOCKDIVISION_DIV1;

htim3.Init.AutoReloadPreload = TIM_AUTORELOAD_PRELOAD_DISABLE;

if (HAL_TIM_Base_Init(&htim3) != HAL_OK) Error_Handler();

sClk.ClockSource = TIM_CLOCKSOURCE_INTERNAL;

if (HAL_TIM_ConfigClockSource(&htim3, &sClk) != HAL_OK) Error_Handler();

sMst.MasterOutputTrigger = TIM_TRGO_RESET;

sMst.MasterSlaveMode     = TIM_MASTERSLAVEMODE_DISABLE;

if (HAL_TIMEx_MasterConfigSynchronization(&htim3, &sMst) != HAL_OK) Error_Handler();

HAL_NVIC_SetPriority(TIM3_IRQn, 2, 0);

HAL_NVIC_EnableIRQ(TIM3_IRQn);

}

static void MX_USART3_UART_Init(void)

{

huart3.Instance          = USART3;

huart3.Init.BaudRate     = 115200;

huart3.Init.WordLength   = UART_WORDLENGTH_8B;

huart3.Init.StopBits     = UART_STOPBITS_1;

huart3.Init.Parity       = UART_PARITY_NONE;

huart3.Init.Mode         = UART_MODE_TX_RX;

huart3.Init.HwFlowCtl    = UART_HWCONTROL_NONE;

huart3.Init.OverSampling = UART_OVERSAMPLING_16;

if (HAL_UART_Init(&huart3) != HAL_OK) Error_Handler();

}

static void MX_USB_OTG_FS_PCD_Init(void)

{

hpcd_USB_OTG_FS.Instance                 = USB_OTG_FS;

hpcd_USB_OTG_FS.Init.dev_endpoints       = 4;

hpcd_USB_OTG_FS.Init.speed               = PCD_SPEED_FULL;

hpcd_USB_OTG_FS.Init.dma_enable          = DISABLE;

hpcd_USB_OTG_FS.Init.phy_itface          = PCD_PHY_EMBEDDED;

hpcd_USB_OTG_FS.Init.Sof_enable          = ENABLE;

hpcd_USB_OTG_FS.Init.low_power_enable    = DISABLE;

hpcd_USB_OTG_FS.Init.lpm_enable          = DISABLE;

hpcd_USB_OTG_FS.Init.vbus_sensing_enable = ENABLE;

hpcd_USB_OTG_FS.Init.use_dedicated_ep1   = DISABLE;

if (HAL_PCD_Init(&hpcd_USB_OTG_FS) != HAL_OK) Error_Handler();

}

static void MX_GPIO_Init(void)

{

GPIO_InitTypeDef GPIO_InitStruct = {0};

__HAL_RCC_SYSCFG_CLK_ENABLE();

__HAL_RCC_GPIOC_CLK_ENABLE();

__HAL_RCC_GPIOH_CLK_ENABLE();

__HAL_RCC_GPIOA_CLK_ENABLE();

__HAL_RCC_GPIOB_CLK_ENABLE();

__HAL_RCC_GPIOD_CLK_ENABLE();

__HAL_RCC_GPIOG_CLK_ENABLE();

/* Initial output levels */

HAL_GPIO_WritePin(GPIOB, LD1_Pin | LD3_Pin | GPIO_PIN_12 | LD2_Pin, GPIO_PIN_RESET);

HAL_GPIO_WritePin(USB_PowerSwitchOn_GPIO_Port, USB_PowerSwitchOn_Pin, GPIO_PIN_RESET);

HAL_GPIO_WritePin(GPIOC, GPIO_PIN_6, GPIO_PIN_RESET);

/* PA3 — button, Falling edge (button to 3.3V, pull-up, press = LOW) */

GPIO_InitStruct.Pin  = BTN_EXT_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_IT_FALLING;

GPIO_InitStruct.Pull = GPIO_PULLUP;

HAL_GPIO_Init(BTN_EXT_GPIO_Port, &GPIO_InitStruct);

/* Ethernet RMII — GPIOC */

GPIO_InitStruct.Pin       = RMII_MDC_Pin | RMII_RXD1_Pin;

GPIO_InitStruct.Mode      = GPIO_MODE_AF_PP;

GPIO_InitStruct.Pull      = GPIO_NOPULL;

GPIO_InitStruct.Speed     = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Alternate = GPIO_AF11_ETH;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/* Ethernet RMII — GPIOA */

GPIO_InitStruct.Pin = RMII_REF_CLK_Pin | RMII_MDIO_Pin | RMII_CRS_DV_Pin;

HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

/* PC4, PC7 — inputs (PC7 = Sensor 1 ECHO) */

GPIO_InitStruct.Pin       = GPIO_PIN_4 | GPIO_PIN_7;

GPIO_InitStruct.Mode      = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull      = GPIO_NOPULL;

GPIO_InitStruct.Alternate = 0;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/* PB0(LD1), PB7(LD2), PB12(Sensor2 TRIG), PB14(LD3) — outputs */

GPIO_InitStruct.Pin   = LD1_Pin | LD3_Pin | GPIO_PIN_12 | LD2_Pin;

GPIO_InitStruct.Mode  = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull  = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

/* Ethernet RMII — RMII_TXD1 */

GPIO_InitStruct.Pin       = RMII_TXD1_Pin;

GPIO_InitStruct.Mode      = GPIO_MODE_AF_PP;

GPIO_InitStruct.Pull      = GPIO_NOPULL;

GPIO_InitStruct.Speed     = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Alternate = GPIO_AF11_ETH;

HAL_GPIO_Init(RMII_TXD1_GPIO_Port, &GPIO_InitStruct);

/* USB PowerSwitch */

GPIO_InitStruct.Pin       = USB_PowerSwitchOn_Pin;

GPIO_InitStruct.Mode      = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull      = GPIO_NOPULL;

GPIO_InitStruct.Speed     = GPIO_SPEED_FREQ_LOW;

GPIO_InitStruct.Alternate = 0;

HAL_GPIO_Init(USB_PowerSwitchOn_GPIO_Port, &GPIO_InitStruct);

/* USB OverCurrent */

GPIO_InitStruct.Pin  = USB_OverCurrent_Pin;

GPIO_InitStruct.Mode = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull = GPIO_NOPULL;

HAL_GPIO_Init(USB_OverCurrent_GPIO_Port, &GPIO_InitStruct);

/* PC6 — Sensor 1 TRIG (output) */

GPIO_InitStruct.Pin   = GPIO_PIN_6;

GPIO_InitStruct.Mode  = GPIO_MODE_OUTPUT_PP;

GPIO_InitStruct.Pull  = GPIO_NOPULL;

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;

HAL_GPIO_Init(GPIOC, &GPIO_InitStruct);

/* Ethernet RMII — GPIOG */

GPIO_InitStruct.Pin       = RMII_TX_EN_Pin | RMII_TXD0_Pin;

GPIO_InitStruct.Mode      = GPIO_MODE_AF_PP;

GPIO_InitStruct.Pull      = GPIO_NOPULL;

GPIO_InitStruct.Speed     = GPIO_SPEED_FREQ_VERY_HIGH;

GPIO_InitStruct.Alternate = GPIO_AF11_ETH;

HAL_GPIO_Init(GPIOG, &GPIO_InitStruct);

/* PB5 — Sensor 2 ECHO (input) */

GPIO_InitStruct.Pin       = GPIO_PIN_5;

GPIO_InitStruct.Mode      = GPIO_MODE_INPUT;

GPIO_InitStruct.Pull      = GPIO_NOPULL;

GPIO_InitStruct.Alternate = 0;

HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);

}

/* USER CODE BEGIN 4 */

void EXTI3_IRQHandler(void) { HAL_GPIO_EXTI_IRQHandler(BTN_EXT_Pin); }

void TIM3_IRQHandler(void)  { HAL_TIM_IRQHandler(&htim3);            }

/* USER CODE END 4 */

void Error_Handler(void)

{

__disable_irq();

while (1) {}

}

#ifdef USE_FULL_ASSERT

void assert_failed(uint8_t *file, uint32_t line) {}

#endif